# BCScanTool — Organized Dataset Capture Plan

## 2011 Toyota Tacoma 4.0L V6 (VIN: 5TFUU4EN2BX005907)

Purpose: Build a production-quality labeled dataset for training fault-detection ML models.
Status: ~110 organic scans captured (unlabeled). Zero controlled fault tests executed.
Scanner: Launch X431 PRO3 V+ Elite (~200 parameters per DataStream capture)

## The Core Problem with Your Current Data

You have ~110 scans but your ML pipeline can't use them effectively because:

- No labels. Your training_pipeline.py infers labels from filenames ('good' in filename). Only one file has "good" in the name, one has "brake". Everything else is unknown.

- No operating condition metadata. You don't know if a scan was cold start, idle, highway, or under load. The same RPM reading means very different things at each.

- Inconsistent parameter coverage. The X431 progressively fills columns as the ECU streams them. Early rows in each CSV are mostly zeros while sensors initialize. Row 1 is useless; row 200 is gold.

- No triplet structure. ML anomaly detection needs baseline → fault → cleared triplets to learn what "deviation from normal" looks like.

This plan fixes all four.

## Part 1: Retroactive Labeling (Do First — 2 Hours)

Before capturing anything new, label what you already have. Create a single metadata file.

### File: data/scan_manifest.csv

| filename | date | condition | operating_mode | engine_temp | notes |
| --- | --- | --- | --- | --- | --- |
| ...20251024151339good_clean.csv | 2024-10-24 | healthy | idle | warm | Labeled "good" by filename |
| ...20251022200941brake_clean.csv | 2024-10-22 | unknown | idle | warm | Brake test? Clarify |
| ...20251025160401 alternation noise_clean.csv | 2024-10-25 | fault_alternator | idle | warm | Real alternator issue |
| ...20251103172406 ac_clean.csv | 2024-11-03 | healthy_ac_on | idle_ac | warm | AC load test |
| ...20260107134353 startup_clean.csv | 2026-01-07 | healthy | cold_start | cold | Startup capture |

For every existing CSV, fill in:

- condition: healthy, fault_alternator, fault_unknown, unknown, post_repair

- operating_mode: cold_start, idle_warm, idle_cold, drive_city, drive_highway, idle_ac, rev_hold_2k, rev_hold_3k

- engine_temp: cold (<160°F), warming (160-190°F), warm (190-210°F), hot (>210°F)

- notes: Anything you remember about that day

This is tedious but it instantly turns 110 unlabeled scans into a usable dataset. You already know which scans were during the alternator issue. Label them.

## Part 2: Baseline Capture Protocol (The Foundation)

Your existing methodology says "3-minute baseline." That's too short and too narrow. A proper baseline needs to cover the operating envelope of the vehicle — the full range of normal behavior across different conditions.

### The 7-Condition Baseline Matrix

Capture one scan for each. Do them all on the same day if possible (weather and conditions consistent).

| # | Condition | How | Duration | Target Rows | Key Parameters to Watch |
| --- | --- | --- | --- | --- | --- |
| B1 | Cold idle | Start engine cold. Don't touch anything. | 5 min | ~600 rows | Coolant temp ramp, STFT swing, O2 sensor warmup, VVT initialization |
| B2 | Warm idle | Engine fully warm (>195°F). No AC. No loads. | 5 min | ~600 rows | Stable RPM, converged fuel trims, O2 switching |
| B3 | Warm idle + AC | AC on max. All else off. | 3 min | ~360 rows | RPM compensation, electrical load, IAC response |
| B4 | Warm idle + electrical load | Headlights, blower, rear defrost all on. | 3 min | ~360 rows | Battery voltage, alternator response, RPM stability |
| B5 | 2000 RPM hold | Hold throttle at steady 2000 RPM in park. | 3 min | ~360 rows | MAF at load, fuel trims under load, spark advance |
| B6 | Drive — city (20-40 mph) | Normal suburban driving. | 5 min | ~600 rows | Throttle response, transmission shifts, speed/load correlation |
| B7 | Drive — highway (55-70 mph) | Steady highway cruise. | 5 min | ~600 rows | High MAF, cruise fuel trims, catalyst temps, VVT at cruise |

### Naming Convention

TACOMA_YYYYMMDD_HHMMSS_B1_cold_idle_baseline.csv

TACOMA_YYYYMMDD_HHMMSS_B2_warm_idle_baseline.csv

TACOMA_YYYYMMDD_HHMMSS_B3_warm_idle_ac_baseline.csv

...

### Why 7 Conditions Matter

A cold-start scan shows fuel trims at +25% and O2 sensors reading 0V — that's normal for open-loop warmup but looks identical to a dead O2 sensor if your model only trained on warm idle. Your model needs to learn: "these parameters are normal for this condition."

### Repeat Schedule

Capture the full B1–B7 matrix:

- Once now (initial baseline)

- Once per month (track seasonal drift — summer vs. winter ambient temps change everything)

- After any repair (new baseline post-intervention)

## Part 3: Controlled Fault Capture Protocol

Your methodology doc has 35+ tests defined. Here's how to prioritize and structure the actual execution.

### Tier 1: High-Value, Low-Risk (Do These First)

These give you the most ML training value with the least risk to the truck.

| Priority | Test | Risk | ML Value | Why First |
| --- | --- | --- | --- | --- |
| 1 | MAF disconnect | 🟢 | VERY HIGH | Easiest test, clearest signature, most common real-world failure |
| 2 | O2 B1S1 disconnect | 🟢 | VERY HIGH | Affects closed-loop fuel control — massive parameter shift |
| 3 | PCV hose disconnect (vacuum leak) | 🟢 | VERY HIGH | Most common real-world issue, affects 10+ parameters |
| 4 | Coolant temp sensor disconnect | 🟢 | HIGH | Changes entire fuel/timing strategy |
| 5 | IAT sensor disconnect | 🟢 | MODERATE | Quick, safe, clean signature |
| 6 | O2 B2S1 disconnect | 🟢 | HIGH | Compare bank-to-bank fault patterns |

### Tier 2: Medium-Value, Medium-Risk (Do After Tier 1 Validates Pipeline)

| Priority | Test | Risk | ML Value |
| --- | --- | --- | --- |
| 7 | TPS disconnect | 🟡 | HIGH |
| 8 | Single coil disconnect (cyl 1) | 🔴 | VERY HIGH |
| 9 | Single injector disconnect (cyl 1) | 🔴 | VERY HIGH |
| 10 | Fuel pressure regulator vacuum disconnect | 🟢 | MODERATE |
| 11 | VVT solenoid disconnect | 🟢 | MODERATE |
| 12 | EGR valve disconnect | 🟢 | MODERATE |

### Tier 3: Multi-Fault (Do Only After Single-Fault Models Work)

| Priority | Test | Risk | ML Value |
| --- | --- | --- | --- |
| 13 | MAF + O2 together | 🟡 | VERY HIGH |
| 14 | Vacuum leak + coil disconnect | 🔴 | VERY HIGH |
| 15 | O2 B1S1 + O2 B2S1 together | 🟢 | HIGH |

### The Triplet Protocol (For Every Single Test)

Each fault test produces exactly 3 files plus a metadata record:

Phase 1: BASELINE     → 3-5 min, engine warm, no faults

File: TACOMA_YYYYMMDD_HHMMSS_T01_maf_disconnect_baseline.csv

Phase 2: FAULT        → Introduce fault, wait 30s to stabilize, then capture 2-3 min

File: TACOMA_YYYYMMDD_HHMMSS_T01_maf_disconnect_fault.csv

Phase 3: CLEARED      → Reconnect, clear DTCs, capture 2 min recovery

File: TACOMA_YYYYMMDD_HHMMSS_T01_maf_disconnect_cleared.csv

Critical rule: The baseline for each test should be captured immediately before the fault, on the same session, same engine temperature. Don't use yesterday's baseline for today's fault — ambient temp, engine condition, and fuel trim adaptations all change.

### Per-Test Metadata Record

After every test, add a row to data/test_executions.csv:

| test_id | date | time | ambient_temp_f | engine_temp_f | mileage | duration_fault_s | dtcs_triggered | symptoms_observed | baseline_file | fault_file | cleared_file | operator_notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T01 | 2026-05-20 | 14:30 | 78 | 198 | 142350 | 150 | P0101,P0102 | High idle, rough, CEL | ...baseline.csv | ...fault.csv | ...cleared.csv | Clean disconnect, immediate response |

## Part 4: What to Capture (Parameter Strategy)

You have ~200 parameters available. Not all are useful for ML.

### Parameter Tiers

Tier A — Always capture, core ML features (25 parameters): These are the parameters that actually change during faults and carry diagnostic signal.

Engine Speed [rpm]

Calculate Load [%]

Coolant Temp [degree C]

Intake Air [degree C]

MAF [gm/s]

Atmosphere Pressure [kPa]

AF Lambda (Bank1 Sensor1)

AF Lambda (Bank2 Sensor1)

AFS Current (Bank1 Sensor1) [mA]

AFS Current (Bank2 Sensor1) [mA]

O2S B1S2 [V]

O2S B2S2 [V]

Short FT (Bank1 Sensor1) [%]

Short FT (Bank2 Sensor1) [%]

Long FT (Bank1 Sensor1) [%]

Long FT (Bank2 Sensor1) [%]

IGN Advance [deg]

All Cylinders Misfire Count

Cylinder Count1-6 Misfire Count

Vehicle Speed [km/h]

Throttle Sensor Position [%]

Accelerator Position [%]

Battery Voltage [V]

Engine Run Time [s]

Tier B — Capture when available, secondary features (20 parameters):

Catalyst Temp (Bank1/2 Sensor1) [degree C]

VVT Change Angle Count1/2 [degree FR]

VVT OCV Duty Count1/2 [%]

Knock Feedback Value [degree CA]

Knock Correct Learn Value [degree CA]

Injector(Port) [us]

Injection Volum (Cylinder1) [ml]

EVAP (Purge) VSV [%]

FC TAU

Fuel Cut Condition

Throttle Motor Current [A]

Throttle Motor DUTY [%]

MISFIRE LOAD [g/rev]

MISFIRE RPM [rpm]

SPD (NT) [rpm]

A/T Oil Temperature 1/2 [degree C]

Tier C — Metadata only, don't feed to ML:

Monitor status flags (Compl/Unable/Not Avl)

MIL status

Count Codes

Distance from DTC Cleared

Model Code, Engine Type, Destination (static)

### Why This Matters

Your autoencoder is currently fed "the first 10 numeric columns" — which might include Row number, monitor completion flags, and static metadata. That's pure noise. Define an explicit feature list and stick to it.

## Part 5: Data Quality Gates

Before any scan enters the training pipeline, it must pass these checks.

### Gate 1: Minimum Row Count

- Baseline scans: ≥300 rows (about 2.5 minutes at 2 Hz)

- Fault scans: ≥120 rows (about 1 minute)

- Reject anything shorter

### Gate 2: Sensor Initialization

- Drop the first 30 rows of every scan. The X431 progressively fills parameters as ECUs respond. Early rows are mostly zeros — not real data, just "haven't heard back yet." Your current pipeline treats these zeros as real sensor readings.

### Gate 3: Engine Running Verification

- RPM > 0 for engine-running tests

- RPM = 0 for key-on-engine-off tests (if you do those)

- Reject scans where RPM drops to zero mid-capture (engine stalled = corrupted data)

### Gate 4: Steady-State Window

- For idle baselines: discard the first 60 seconds of warm-up ramp

- For fault captures: discard the first 30 seconds after fault introduction (transient response)

- The ML model should learn the settled fault signature, not the transition

### Gate 5: Metadata Complete

- Every file in scan_manifest.csv or test_executions.csv

- No orphaned files

## Part 6: Dataset Directory Structure

Reorganize your data directory to separate concerns:

data/

├── raw/                          # Untouched scanner output (.x431, .pdf)

│   └── Toyota/Tacoma/

├── csv/                          # Converted CSVs (all scans, converted from .x431)

│   └── Toyota/Tacoma/

├── labeled/                      # The ML-ready dataset

│   ├── baselines/                # B1-B7 baseline captures

│   │   ├── TACOMA_20260520_B1_cold_idle.csv

│   │   ├── TACOMA_20260520_B2_warm_idle.csv

│   │   └── ...

│   ├── faults/                   # Controlled fault captures (triplets)

│   │   ├── T01_maf_disconnect/

│   │   │   ├── baseline.csv

│   │   │   ├── fault.csv

│   │   │   └── cleared.csv

│   │   ├── T02_o2_b1s1_disconnect/

│   │   │   ├── baseline.csv

│   │   │   ├── fault.csv

│   │   │   └── cleared.csv

│   │   └── ...

│   ├── organic/                  # Your existing 110 scans, labeled retroactively

│   │   ├── healthy/

│   │   ├── fault_alternator/

│   │   └── unknown/

│   └── multi_fault/              # Combined fault scenarios

│       └── T13_maf_plus_o2/

├── metadata/

│   ├── scan_manifest.csv         # Retroactive labels for organic data

│   ├── test_executions.csv       # Controlled test metadata

│   └── parameter_tiers.json      # Which columns are Tier A/B/C

└── processed/                    # Pipeline output (features, predictions)

## Part 7: Execution Timeline

### Week 1: Foundation

- Create scan_manifest.csv — label all 110 existing scans

- Create parameter_tiers.json — define Tier A/B/C columns

- Update training_pipeline.py to read labels from manifest instead of filenames

- Implement Gate 1-5 preprocessing in the pipeline

### Week 2: Baseline Matrix

- Capture B1 through B7 (one session, ~35 min total scan time)

- Verify all 7 files pass quality gates

- Retrain ML models on labeled organic + baseline data

- Record baseline model performance as your benchmark

### Week 3: First Fault Tests (Tier 1, tests 1-3)

- T01: MAF disconnect (triplet)

- T02: O2 B1S1 disconnect (triplet)

- T03: PCV vacuum leak (triplet)

- Retrain after each test, measure detection accuracy

### Week 4: Remaining Tier 1 (tests 4-6)

- T04: Coolant temp sensor disconnect

- T05: IAT sensor disconnect

- T06: O2 B2S1 disconnect

- Full model retrain, cross-validate on all fault types

### Month 2: Tier 2 + Repetition

- Execute Tier 2 tests (T07-T12)

- Repeat Tier 1 tests at different engine temps (cold vs. warm)

- Repeat B1-B7 baseline matrix (second month)

### Month 3: Multi-Fault + Validation

- Tier 3 combined fault tests

- Hold-out validation set: re-run 3 random faults, don't train on them, use as blind test

- Publish results in model performance table

## Part 8: Sample Size Targets

For supervised learning to work with your Random Forest, you need:

| Class | Minimum Samples | Target Samples | Current |
| --- | --- | --- | --- |
| healthy (various modes) | 30 | 100+ | ~5 (labeled) |
| fault_maf | 5 | 15 | 0 |
| fault_o2 | 5 | 15 | 0 |
| fault_vacuum | 5 | 15 | 0 |
| fault_ect | 5 | 15 | 0 |
| fault_misfire | 5 | 15 | 0 |
| fault_alternator | 3 | 10 | ~3 (unlabeled) |
| multi_fault | 3 | 10 | 0 |

How to get 15 samples per fault type without doing the test 15 times:

- Window slicing. A 3-minute fault scan at 2 Hz = 360 rows. Slice it into non-overlapping 60-row windows → 6 samples per scan.

- Condition variation. Do the same fault at cold idle, warm idle, and 2000 RPM → 3 scans × 6 windows = 18 samples.

- Temporal repetition. Repeat the same test a month later → another 18 samples with natural variation.

This gets you to 30+ samples per fault type from just 6 actual test sessions per fault.

## Part 9: What This Enables in Your ML Pipeline

Once you have this structured data, your pipeline changes from:

# CURRENT: guess labels from filenames

if 'good' in filename:

df['condition'] = 'good'

To:

# FUTURE: read from manifest

manifest = pd.read_csv('data/metadata/scan_manifest.csv')

for _, row in manifest.iterrows():

df = pd.read_csv(row['filepath'])

df = df.iloc[30:]  # Gate 2: drop initialization rows

df = df[TIER_A_COLUMNS]  # Only ML-relevant features

df['condition'] = row['condition']

df['operating_mode'] = row['operating_mode']

all_dfs.append(df)

And your LSTM gets multivariate input instead of single-sensor:

# FUTURE: train on Tier A features as multivariate time series

features = df[TIER_A_COLUMNS].values  # shape: (timesteps, 25)

# Instead of training separate models per sensor

## Quick-Reference: Test Day Checklist

□ Scanner charged, cable connected

□ Clear all existing DTCs before starting

□ Note ambient temperature and mileage

□ Engine at target temperature for test type

□ Phone camera ready for documentation

For each test:

□ Start X431 DataStream recording (ALL parameters)

□ Capture BASELINE (3-5 min)

□ Save file with correct naming convention

□ Introduce fault

□ Wait 30s for stabilization

□ Capture FAULT (2-3 min)

□ Save file

□ Restore component

□ Clear DTCs

□ Capture CLEARED (2 min)

□ Save file

□ Fill in test_executions.csv row

□ Take photo of connector/component tested

After all tests:

□ Verify all files saved correctly

□ Transfer to MacBook

□ Run quality gate checks

□ Commit to git with test ID in commit message